Accelerator-Based Epithermal Neutron Source for BNCT

6105 Coronado Ave. NE                     Albuquerque, NM  87109                      Website:  www.linac.com

Phone: (505) 797-5614                            Fax: (505) 797-5615                       Email: linacinfo@linac.com

Packaging (Linac Cabinet):

Length		2.5	m

Width		0.7	m

Height (beam line)	1.2 	m

Weight 		1000 	kg

Packaging (RF Power):

Electronic Racks	2	racks

Utilities:

Electrical Power	260 	kVA

Accelerator Cooling	70	kW

Target Cooling	50	kW

Neutron Beam Options:

Solid Lithium Target

Moderator Assembly

Epithermal Neutron Flux	      2.2X109  n/cm2/s

Features:

ECR Microwave Ion Source

Magnetic solenoid lens LEBT

RFQ Linac to 0.75 MeV

RFI Linac to 2.5 MeV

Resonantly Coupled Linacs

Single Rf Power Drive Point

Tetrode Rf Power System

Parameters:

Beam Particle	Proton

Energy		2.5 	MeV

Rf Frequency	200 	MHz

Beam Current 	(peak)	20 	mA

(average)	20 	mA

Beam Duty Factor (CW)	100	%

Beam Power (CW)	50	kW

Rf Cavity Power (CW)	70	kW

Rf Power (total)	120 	kW

An economical Accelerator-Based Epithermal Neutron Source (ABENS) for the BNCT application!  Based on the latest development in linac technology, the revolutionary RFI Linac Structure, this unit offers higher efficiency operation from smaller units at lower cost.  While still under development, these projections are based on well-established accelerator development techniques.  To get the latest information on this and other related devices, please visit our website: www.linac.com.